# PEC GET PROGRAM

# NECOP INTERNSHIP

# IC SYSTEM DESIGN VERIFICATION

# EDA PLAYGROUND AND VERIFICATION GUIDE

# SUMBITTED BY

HUZAIFA SHAHBAZ

# SUMBITTED TO

SHAHBAZ AHMAD

DEPARTMENT

IC SYSTEM DESIGN AND VERIFICATION

DATE:

28th April 2026

# EDA PLAYGROUND

In today’s work of my PEC internship of NECOP was to make EDA playground account to login with my personal email account for the ic system design and verification fabrication. To see that how it works and how it waveforms generates in the system Verilog and the next part which task given me is that verification guide of system Verilog tutorial and run its code on the eda playground.

# VERIFICATION GUIDE OF SYSTEM VERLIOG

System Verilog Datatypes

Structural Data Types

wire and reg

structural data types called nets, which model hardware connections between circuit components. The wire nets act like real wires in circuits. The reg type holds their values until another value is put on them, just like a register hardware component.

The declarations for wire and reg signals are inside a module but outside any initial or always block.

The initial state of a,

reg is x unknown, wire is z.

Behavioral Data Types

integer, real, and time

An integer declares one or more variables of type integer. These variables can hold values ranging from -231 to (231)-1.

Integer Syntax:

integer integer_variable_name;

A real declaration declares one or more variables of type real. The real variables are stored as 64-bit quantities and store the real values. Real numbers can be specified in either decimal notation (for example, 14.72) or in scientific notation (for example, 39e8).

Examples:

1.8 635.16

1.2E12 (the exponent symbol can be e or E)

1.30e-2

Syntax

real real_variable_name;

Default value of integer type variable is “x” and Default value of real type variable is “0”.

Examples:

integer a[0:64] ; // an array of 65 integer values

real float_v ; // a variable to store real value

Time

Time is a 64-bit quantity that can be used in conjunction with the $time system task to hold simulation time. Time is not supported for synthesis and hence is used only for simulation purposes.

Syntax

time time_variable_name;

Parameters

Parameters represent constants; hence it is illegal to modify their value at runtime. However, parameters can be modified at compilation time to have values that are different from those specified in the declaration assignment. This allows the customization of module instances. A parameter can be modified with the defparam statement, or in the module instance statement.

parameter size = 16 ;

logic

logic is the improved version of reg form Verilog to SystemVerilog, so it Can be driven by continuous assignments, gates, and modules in addition to being a variable. declaration:

bit                             – Unsigned

byte, shortint, int, longint    – Signed

unsigned two-state types,

bit            single_bit ;    // unsigned single bit

bit [31:0]     32_bit      ;    // 32-bit unsigned integer

signed two-state types,

int           integer;   // 32-bit signed integer

byte          8_bit  ;   //   8-bit signed integer

shortint      16_bit ;   // 16-bit signed integer

longint       64_bit ;   // 64-bit signed integer

unsigned from signed two-state types,

int           unsigned integer;   // 32-bit unsigned integer

byte          unsigned  8_bit;   //   8-bit unsigned integer

shortint      unsigned  16_bit;   // 16-bit unsigned integer

longint       unsigned  64_bit;   // 64-bit unsigned integer

Void Data Types

The void data type represents non-existent data. This type can be specified as the return type of functions, indicating no return value.

syntax:

void’(function_call());

String

A string data type is variable size, it is a dynamically allocated array of bytes.

string declaration examples:

string             s1    =   "Hellow World";

string             s2    =   {"Hi"," ",s1};

bit [31:0]         b      =   128;

string             s3    =   b;      // sets 128 to s3

module string_datatype;

//declaration

string s1 = "Hello World";

string s2 = {"Hi,"," ",s1};

bit[31:0]b= 128;

string s3 = b; // sets 128 to s3

initial begin

//display values

$display("String 1 s1 = %0s",s1);

$display("String 2 s2 = %0s",s2);

$display("String 3 s3 = %0d bit b = %0d",s3,b);

end

endmodule

Simulator Output

String 1 s1 = Hello World

String 2 s2 = Hi, Hello World

String 3 s3 = 128 bit b = 128

CODE ON EDA PLAYGROUND:

// Code your design here

//-------------------------------------------------------------------------

// www.verificationguide.com - Data Types (String Example)

//-------------------------------------------------------------------------

module string_datatype;

// Declaration

string      s1 = "Hello World";

string      s2 = {"Hi,", " ", s1};  // string concatenation

bit [31:0]  b  = 128;

string      s3;

initial begin

// Assign integer to string (ASCII conversion)

s3 = b;

// Display values

$display("String 1 s1 = %0s", s1);

$display("String 2 s2 = %0s", s2);

$display("String 3 s3 = %0s, bit b = %0d", s3, b);

end

endmodule

OUTPUT ON EDA PLAYGROUND:

Figure 1:OUTPUT OF String DATA TYPES OF SYSTEM VERLIOG

EVENT

System Verilog enhances the Verilog event in several ways. An event is now a handle to a synchronization object that can be passed around to routines. In Verilog, if the triggering thread executes before the blocking thread, the trigger is missed. System Verilog introduces triggered function that lets you check whether an event has been triggered.

event declaration examples,

event  e1;

event  e2;

event  done;

User Defined

The user can define a new type using typedef, as in C. This can then be instantiated as,

integer_v var1;

integer_v var2;

System Verilog Enumerations

System Verilog enum data type

An enumerated type defines a set of named values. The simplest enumerated type declaration contains a list of constant names and one or more variables.

In the following example, colors are defined to be variable of the unnamed enumerated int type that includes the members red, green, blue, yellow, white, black.

enum { red, green, blue, yellow, white, black } Colors;

The actual values are defaulted to integers starting at 0 and then increase. in the above example by default variable will get the default value of 0,1,2,3,4,5 respectively from red. The values can be set for the names and also values can be set for some of the names and not set for other names. A name without a value is automatically assigned an increment of the value of the previous name.

In the following example value is set for red = 0, blue = 4, white = 10.

enum { red=0, green, blue=4, yellow, white=10, black } Colors;

green, yellow, black automatically assigned to the increment-value of 1,5,11 respectively.

If an automatically incremented value is assigned elsewhere in the same enumeration, this shall be a syntax error.

In the below example yellow will get the increment-value of 5, the value of white is set with 5. this will cause the syntax error.

enum { red=0, green=0, blue=4, yellow, white=5, black=6 } Colors

Defining new data types as enumerated types

A type name can be given so that the same type can be used in many places.

typedef enum {GOOD, BAD} pkt_type;

pkt_type pkt_a; // named type

enum methods

| Method | Description |
| --- | --- |
| first() | returns the value of the first member of the enumeration |
| last() | returns the value of the last member of the enumeration |
| next() | returns the value of next member of the enumeration |
| next(N) | returns the value of next Nth member of the enumeration |
| prev() | returns the value of previous member of the enumeration |
| prev(N) | returns the value of previous Nth member of the enumeration |
| num() | returns the number of elements in the given enumeration |
| name() | returns the string representation of the given enumeration value |

enum examples

Example-1 : Enumeration Type [DataTypes] This example shows how to declare enum.

// Code your design here

//----------------------------------------------------------------------//

//              example2 : Enumeration Type [DataTypes]			        //

//	This example shows how to declare enum, how to use the enumeration	//

//					methods first and name 						//

//----------------------------------------------------------------------//

module enum_datatype;

//declaration

enum { red, green, blue, yellow, white, black } Colors;

//display members of Colors

initial begin

Colors = Colors.first;

for(int i=0;i<6;i++) begin

$display("Colors :: Value of  %0s \t is = %0d",Colors.name,Colors);

Colors = Colors.next;

end

end

endmodule

OUTPUT:

Example-2 : Enumeration Type [DataTypes]

This example shows how to set other than default values to an enum.

// Code your design here

//----------------------------------------------------------------------//

//              example3 : Enumeration Type [DataTypes]			        //

//   example shows how to set values to enum other than default values  //

//----------------------------------------------------------------------//

module enum_datatype;

//declaration

enum { red=0, green, blue=4, yellow, white=10, black } Colors;

//display members of Colors

initial begin

Colors = Colors.first;

for(int i=0;i<6;i++) begin

$display("Colors :: Value of  %0s \t is = %0d",Colors.name,Colors);

Colors = Colors.next;

end

end

endmodule

OUTPUT:

Example-3 : Enumeration Type [DataTypes]

This example shows an error, in case of automatic increment-value, is the same as the value assigned to another enum member.

// Code your design here

module enum_datatype;

//declaration

enum { red=0, green=1, blue=4, yellow, white=5, black=6 } Colors;

initial begin

Colors = Colors.first;

for(int i=0;i<6;i++)

$display("Colors  ::  Value of  %0s is = %0d",Colors.name(),Colors);

end

endmodule

OUTPUT:

Example-4 : Enumeration Type [DataTypes]

This example shows declaring new data types using the enumerated type.

// Code your design here

module enum_datatype;

//declaration

typedef enum {GOOD, BAD} pkt_type;

pkt_type  pkt_a;

pkt_type  pkt_b;

initial begin

pkt_a = GOOD;

pkt_b=BAD;

if(pkt_a == GOOD)

$display("pkt_a is  GOOD packet");

else

$display("pkt_a is  BAD  packet");

if(pkt_b == GOOD)

$display("pkt_b is  GOOD packet");

else

$display("pkt_b is  BAD  packet");

end

endmodule

OUTPUT: