# PEC GET PROGRAM

# NECOP INTERNSHIP

# IC SYSTEM DESIGN VERIFICATION

# VERIFICATION GUIDE OF SYSTEM VERLIOG

# SUMBITTED BY

HUZAIFA SHAHBAZ

# SUMBITTED TO

SHAHBAZ AHMAD

DEPARTMENT

IC SYSTEM DESIGN AND VERIFICATION

DATE:

29th April 2026

System Verilog Datatypes

Structural Data Types

wire and reg

structural data types called nets, which model hardware connections between circuit components. The wire nets act like real wires in circuits. The reg type holds their values until another value is put on them, just like a register hardware component.

The declarations for wire and reg signals are inside a module but outside any initial or always block.

The initial state of a,

reg is x unknown, wire is z.

Behavioral Data Types

integer, real, and time

An integer declares one or more variables of type integer. These variables can hold values ranging from -231 to (231)-1.

Integer Syntax:

integer integer_variable_name;

A real declaration declares one or more variables of type real. The real variables are stored as 64-bit quantities and store the real values. Real numbers can be specified in either decimal notation (for example, 14.72) or in scientific notation (for example, 39e8).

Examples:

1.8 635.16

1.2E12 (the exponent symbol can be e or E)

1.30e-2

Syntax

real real_variable_name;

Default value of integer type variable is “x” and Default value of real type variable is “0”.

Examples:

integer a[0:64] ; // an array of 65 integer values

real float_v ; // a variable to store real value

Time

Time is a 64-bit quantity that can be used in conjunction with the $time system task to hold simulation time. Time is not supported for synthesis and hence is used only for simulation purposes.

Syntax

time time_variable_name;

Parameters

Parameters represent constants; hence it is illegal to modify their value at runtime. However, parameters can be modified at compilation time to have values that are different from those specified in the declaration assignment. This allows the customization of module instances. A parameter can be modified with the defparam statement, or in the module instance statement.

parameter size = 16 ;

logic

logic is the improved version of reg form Verilog to SystemVerilog, so it Can be driven by continuous assignments, gates, and modules in addition to being a variable. declaration:

bit                             – Unsigned

byte, shortint, int, longint    – Signed

unsigned two-state types,

bit            single_bit ;    // unsigned single bit

bit [31:0]     32_bit      ;    // 32-bit unsigned integer

signed two-state types,

int           integer;   // 32-bit signed integer

byte          8_bit  ;   //   8-bit signed integer

shortint      16_bit ;   // 16-bit signed integer

longint       64_bit ;   // 64-bit signed integer

unsigned from signed two-state types,

int           unsigned integer;   // 32-bit unsigned integer

byte          unsigned  8_bit;   //   8-bit unsigned integer

shortint      unsigned  16_bit;   // 16-bit unsigned integer

longint       unsigned  64_bit;   // 64-bit unsigned integer

Void Data Types

The void data type represents non-existent data. This type can be specified as the return type of functions, indicating no return value.

syntax:

void’(function_call());

String

A string data type is variable size, it is a dynamically allocated array of bytes.

string declaration examples:

string             s1    =   "Hellow World";

string             s2    =   {"Hi"," ",s1};

bit [31:0]         b      =   128;

string             s3    =   b;      // sets 128 to s3

CODE OF EDA PLAYGROUND:

// Code your design here

module string_datatype;

//declaration

string s1 = "Hello World";

string s2 = {"Hi,"," ",s1};

bit[31:0]b= 128;

string s3 = b; // sets 128 to s3

initial begin

//display values

$display("String 1 s1 = %0s",s1);

$display("String 2 s2 = %0s",s2);

$display("String 3 s3 = %0d bit b = %0d",s3,b);

end

endmodule

OUTPUT:

EVENT

System Verilog enhances the Verilog event in several ways. An event is now a handle to a synchronization object that can be passed around to routines. In Verilog, if the triggering thread executes before the blocking thread, the trigger is missed. System Verilog introduces triggered function that lets you check whether an event has been triggered.

event declaration examples,

event  e1;

event  e2;

event  done;

User Defined

The user can define a new type using typedef, as in C. This can then be instantiated as,

integer_v var1;

integer_v var2;

System Verilog Enumerations

System Verilog enum data type

An enumerated type defines a set of named values. The simplest enumerated type declaration contains a list of constant names and one or more variables.

In the following example, colors are defined to be variable of the unnamed enumerated int type that includes the members red, green, blue, yellow, white, black.

enum { red, green, blue, yellow, white, black } Colors;

The actual values are defaulted to integers starting at 0 and then increase. in the above example by default variable will get the default value of 0,1,2,3,4,5 respectively from red. The values can be set for the names and also values can be set for some of the names and not set for other names. A name without a value is automatically assigned an increment of the value of the previous name.

In the following example value is set for red = 0, blue = 4, white = 10.

enum { red=0, green, blue=4, yellow, white=10, black } Colors;

green, yellow, black automatically assigned to the increment-value of 1,5,11 respectively.

If an automatically incremented value is assigned elsewhere in the same enumeration, this shall be a syntax error.

In the below example yellow will get the increment-value of 5, the value of white is set with 5. this will cause the syntax error.

enum { red=0, green=0, blue=4, yellow, white=5, black=6 } Colors

Defining new data types as enumerated types

A type name can be given so that the same type can be used in many places.

typedef enum {GOOD, BAD} pkt_type;

pkt_type pkt_a; // named type

enum methods

| Method | Description |
| --- | --- |
| first() | returns the value of the first member of the enumeration |
| last() | returns the value of the last member of the enumeration |
| next() | returns the value of next member of the enumeration |
| next(N) | returns the value of next Nth member of the enumeration |
| prev() | returns the value of previous member of the enumeration |
| prev(N) | returns the value of previous Nth member of the enumeration |
| num() | returns the number of elements in the given enumeration |
| name() | returns the string representation of the given enumeration value |

enum examples

Example-1 : Enumeration Type [DataTypes] This example shows how to declare enum.

CODE OF EDA PLAYGROUND:

// Code your design here

module enum_datatype;

//declaration

enum { red, green, blue, yellow, white, black } Colors;

//display members of Colors

initial begin

Colors = Colors.first;

for(int i=0;i&amp;amp;amp;amp;amp;amp;amp;amp;amp;lt;6;i++) begin

$display("Colors :: Value of  %0s \t is = %0d",Colors.name,Colors);

Colors = Colors.next;

end

end

endmodule

OUTPUT:

Example-2 : Enumeration Type [DataTypes]

This example shows how to set other than default values to an enum.

CODE OF EDA PLAYGROUND:

//----------------------------------------------------------------------//

//              example3 : Enumeration Type [DataTypes]			        //

//   example shows how to set values to enum other than default values  //

//----------------------------------------------------------------------//

module enum_datatype;

//declaration

enum { red=0, green, blue=4, yellow, white=10, black } Colors;

//display members of Colors

initial begin

Colors = Colors.first;

for(int i=0;i<6;i++) begin

$display("Colors :: Value of  %0s \t is = %0d",Colors.name,Colors);

Colors = Colors.next;

end

end

endmodule

OUTPUT:

Example-3 : Enumeration Type [DataTypes]

This example shows an error, in case of automatic increment-value, is the same as the value assigned to another enum member.

CODE OF EDA PLAYGROUND:

// Code your design here

//-------------------------------------------------------------------------

//				www.verificationguide.com - Enumeration Type [DataTypes]

//-------------------------------------------------------------------------

module enum_datatype;

//declaration

enum { red=0, green=1, blue=4, yellow, white=5, black=6 } Colors;

initial begin

Colors = Colors.first;

for(int i=0;i<6;i++)

$display("Colors  ::  Value of  %0s is = %0d",Colors.name(),Colors);

end

endmodule

OUTPUT:

Example-4 : Enumeration Type [DataTypes]

This example shows declaring new data types using the enumerated type.

CODE OF EDA PLAYGROUND:

//-------------------------------------------------------------------------

//				www.verificationguide.com - Enumeration Type [DataTypes]

//-------------------------------------------------------------------------

module enum_datatype;

//declaration

typedef enum {GOOD, BAD} pkt_type;

pkt_type  pkt_a;

pkt_type  pkt_b;

initial begin

pkt_a = GOOD;

pkt_b=BAD;

if(pkt_a == GOOD)

$display("pkt_a is  GOOD packet");

else

$display("pkt_a is  BAD  packet");

if(pkt_b == GOOD)

$display("pkt_b is  GOOD packet");

else

$display("pkt_b is  BAD  packet");

end

endmodule

OUTPUT:

Example-5 : Enumeration Type [DataTypes]

This example shows how to use enumeration methods.

CODE OF EDA PLAYGROUND:

// Code your design here

//-------------------------------------------------------------------------

//				www.verificationguide.com - Enumeration Type [DataTypes]

//-------------------------------------------------------------------------

module enum_datatype;

//declaration

typedef enum { red=0, green, blue=4, yellow, white=10, black } colors;

enum { a, b, c, d, e, f, g } alphabets;

colors first_set;

colors second_set;

initial begin

first_set = first_set.first();

$display("first_set first color is \t %0s, \t Value = %0d", first_set.name(),first_set);

first_set = first_set.last();

$display("first_set last color is \t %0s, \t Value = %0d", first_set.name(),first_set);

second_set = first_set;

$display("second_set color is \t %0s, \t Value = %0d", second_set.name(),second_set);

second_set =second_set.prev(2);

$display("second_set color is \t %0s, \t Value = %0d", second_set.name(),second_set);

second_set =second_set.next(2);

$display("second_set color is \t %0s, \t Value = %0d", second_set.name(),second_set);

$display("Number of members in alphabets is \t %0d",alphabets.num());

$display("Default First members in alphabets is \t %0s , \t value is %0d",alphabets.name(),alphabets);

alphabets=alphabets.next;

$display("Next members in alphabets is \t %0s , \t value is %0d",alphabets.name(),alphabets);

alphabets=alphabets.last;

$display("Last members in alphabets is \t %0s , \t value is %0d",alphabets.name(),alphabets);

alphabets=alphabets.prev(3);

$display("3rd members from last in alphabets is \t %0s , \t value is %0d",alphabets.name(),alphabets);

end

endmodule

OUTPUT:

class data type

A Class is a collection of data and a set of subroutines that operate on that data. The data in a class are referred to as class properties, and its subroutines are called methods. A Class is declared using the class…endclass keywords.

Declaration Example:

class packet;

// Properties

bit [31:0] address;

bit [31:0] data   ;

// Method

function new();

$display("Inside new Function of packet");

endfunction

endclass : packet

System Verilog Fixed size Array

Array in SystemVerilog

Array

An array is a collection of variables, all of the same type, and accessed using the same name plus one or more indices. there are different types of arrays, few array declaration examples are given below.

Example:

int array1 [6];            //fixed size single dimension array

int array2 [5:0];          //fixed size single dimension array

int array3 [3:0][2:0];     //fixed size multi dimension array

bit [7:0] array4[2:0];     //unpacked array declaration

bit [2:0][7:0] array5;     //packed array declaration

bit [2:0][7:0] array6 [3]; //mixed packed and unpacked array

In SystemVerilog vector width/dimensions declared before the object name is referred to as packed array and array size/dimensions declared after the object name is referred to as an unpacked array.

A packed array is a mechanism for subdividing a vector into sub-fields which can be conveniently accessed as array elements.

Fixed size array

In fixed size array, array size will be constant throughout the simulation, Once the array is declared no need to create it. By default, the array will be initialized with value ‘0’.

Single dimensional array

int array1 [6];   //Compact declaration

int array2 [5:0]; // Verbose declaration

Multidimensional array

Multidimensional arrays are also known as an array of arrays.

Two-dimensional array

int arr[2][3];

This array has total 2*3 = 6 elements.

Three-dimensional array

int arr[2][2][2];

This array has total 2*2*2 = 8 elements.

Two-dimensional array declaration

int array3 [2:0][3:0];

The data in a two-dimensional array is stored in a tabular form as shown in the below diagram.

array assignment

array1 = '{0,1,2,3,4,5};

array2 = '{0,1,2,3,4,5};

array3 = '{ '{0,1,2,3},'{4,5,6,7},'{8,9,10,11}};

Fixed Size Array Example

This example shows array declaration and array manipulation using for and foreach loop.

CODE OF EDA PLAYGROUND:

// Code your design here

//-------------------------------------------------------------------------

//						www.verificationguide.com

//                      Fixed Size array

//-------------------------------------------------------------------------

module fixedsize_array;

//declaration of array’s

int array1[6];               //single dimension array

int array2[5:0];             //single dimension array

int array3[2:0][3:0];        //multi dimension array

int array4[4:0];

initial begin

//array initialization

array1 = '{0,1,2,3,4,5};

array2 = '{0,1,2,3,4,5};

array3 = '{'{0,1,2,3},'{4,5,6,7},'{8,9,10,11}};

//displaying array elements

$display("-------displaying array1-------");

foreach(array1[i]) $display("\t array1[%0d] = %0d",i,array1[i]);

$display("-------displaying array2-------");

for(int i=0;i<6;i++) $display("\t array2[%0d] = %0d",i,array2[i]);

$display("-------displaying array3-------");

foreach(array3[i,j]) $display("\t array3[%0d][%0d] = %0d",i,j,array3[i][j]);

$display("-------displaying uninitialized array4-------");

for(int i=0;i<5;i++) $display("\t array4[%0d] = %0d",i,array4[i]);

end

endmodule

OUTPUT:

[2026-04-29 18:33:34 UTC] verilator --prefix Vsim --binary --build-jobs 0 '--build' '--trace-vcd' '--quiet-build' '-Wno-fatal' '--timescale' '1ns/1ns' design.sv testbench.sv  && obj_dir/Vsim  
- V e r i l a t i o n   R e p o r t: Verilator 5.044 2026-01-01 rev v5.044
- Verilator: Built from 0.043 MB sources in 2 modules, into 0.035 MB in 9 C++ files needing 0.000 MB
- Verilator: Walltime 8.494 s (elab=0.001, cvt=0.011, bld=8.470); cpu 0.019 s on 8 threads; alloced 30.273 MB
-------displaying array1-------
	 array1[0] = 0
	 array1[1] = 1
	 array1[2] = 2
	 array1[3] = 3
	 array1[4] = 4
	 array1[5] = 5
-------displaying array2-------
	 array2[0] = 5
	 array2[1] = 4
	 array2[2] = 3
	 array2[3] = 2
	 array2[4] = 1
	 array2[5] = 0
-------displaying array3-------
	 array3[2][3] = 0
	 array3[2][2] = 1
	 array3[2][1] = 2
	 array3[2][0] = 3
	 array3[1][3] = 4
	 array3[1][2] = 5
	 array3[1][1] = 6
	 array3[1][0] = 7
	 array3[0][3] = 8
	 array3[0][2] = 9
	 array3[0][1] = 10
	 array3[0][0] = 11
-------displaying uninitialized array4-------
	 array4[0] = 0
	 array4[1] = 0
	 array4[2] = 0
	 array4[3] = 0
	 array4[4] = 0
- S i m u l a t i o n   R e p o r t: Verilator 5.044 2026-01-01
- Verilator: end at 0s; walltime 0.000 s; speed 0.000 s/s
- Verilator: cpu 0.000 s on 1 threads; alloced 6 MB
Done