# Verification of APB Slave Verification plan

Table 1: Verification of APB Slave Verification plan

| # | Test Name | Feature Verified |
| --- | --- | --- |
| 1 | Reset Test | Proper reset behavior; all signals return to default values |
| 2 | Single Write Transfer | Valid write transaction: PSEL, PENABLE, PWRITE = 1, correct data acceptance |
| 3 | Single Read Transfer | Valid read transaction: PSEL, PENABLE, PWRITE = 0, correct data return |
| 4 | Back to Back Transfers | Handling consecutive transfers without violating setup/access phase rules |
| 5 | Wait State Insertion | Slave uses PREADY to extend access phase correctly |
| 6 | Error Response Test | PSLVERR asserted for invalid access or unsupported address |
| 7 | Address Decoding Test | Correct response to valid address range; PSLVERR for invalid addresses |
| 8 | Write to Read Transition | Proper handling of consecutive write followed by read |
| 9 | Read to Write Transition | Proper handling of consecutive read followed by write |
| 10 | Idle State Test | Slave remains stable when PSEL = 0 (no active transfer) |
| 11 | Signal Stability Test | Signals remain stable during setup and access phase as required |
| 12 | Timing Constraint Test | Verification of setup/hold times and protocol timing requirements |
| 13 | Multiple Sequential Accesses | Robustness under repeated accesses (burst-like simulation, though APB is non-burst) |
| 14 | Corner Case Test | Handling of simultaneous error/wait state scenarios |