# Mercury sensor core monitoring and capabilities of water purification devices

Table 1: Core monitoring and capabilities of mercury sensors of water purification devices

| Function | Why it is important | Considerations |
| --- | --- | --- |
| Mercury Detection / Sensing | Detect concentration of mercury ions (Hg²⁺) in water, reliably (e.g. in parts per billion / ppm) | Electrochemical sensor probe (e.g. modified electrode) that responds to Hg. Optical / colorimetric module if using reagents that change color in presence of Hg. Custom nanosensors or sensor material (e.g. functionalized electrode surface).Calibrate with standard solutions |
| Other Water-Quality Monitoring | pH, temperature, turbidity, total dissolved solids (TDS), conductivity, flow rate | Use sensors for these parameters to monitor general water quality; also, useful to check whether your removal is harming other metrics or to trigger when removal unit needs regeneration / maintenance |
| Data Acquisition & Logging | Read sensors periodically, process raw values, filter noise, compute average / trending | Use ESP32-S3 ADC / interface (I2C / SPI / analog) to read sensors; buffer / smoothing; store to local flash or send to cloud |
| Communication / Reporting | Send data remotely / locally; alert user if mercury levels exceed thresholds; dashboard / app integration | Use WiFi / BLE (ESP32-S3 supports both) to upload data to server / cloud / IoT dashboard; maybe MQTT / HTTP / REST API; also, possibly mobile app notifications |
| Control / Actuation | Trigger removal process (pump on/off / valve actuation / regeneration cycle) automatically when high mercury detected | Use GPIO / relays / pumps / solenoid valves; logic on ESP32 to decide when to flush, when to regenerate adsorbent / change filter medium |
| Removal Mechanism | How to remove mercury once it's detected | Possible methods: • Adsorbent cartridge (e.g. activated carbon, MOF-based material) • Regeneration capability (flush with chemical reagent) • Flow-through membrane / filter setup • Possibly real-time flow rate regulation to ensure sufficient contact time |
| Regeneration / Maintenance Monitoring | Monitor when adsorbent is saturated; track number of cycles; initiate regeneration; log performance degradation over time | Sensor for measuring inflow vs outflow mercury concentration; track adsorption efficiency over cycles; flag when removal rate drops under threshold |
| Safety & Calibration Features | Safe chemical handling; calibrate sensors; verify accuracy; implement fail-safe if high mercury persists | Include calibration routines; check sensor drift; safe shutdown if contaminant level stays high |

Research paper and its links

- Removal of mercury from water by phytoremediation

Link: https://pmc.ncbi.nlm.nih.gov/articles/PMC10404195

- Rapid Removal of Mercury from Water by Novel MOF/PP Hybrid Membrane

Link: https://www.mdpi.com/2079-4991/11/10/2488

- Progress on Mercury-containing Wastewater Treatment

Link: https://drpress.org/ojs/index.php/HSET/article/view/13101

- Mercury Removal from Water Using a Novel Composite of Polyacrylate-Modified Carbon

Link: https://www.researchgate.net/publication/360026821_Mercury_Removal_from_Water_Using_a_Novel_Composite_of_Polyacrylate-Modified_Carbon

- Water Quality Monitoring Using Internet of Things (IOT) and Machine Learning (ML) for Domestic Application

Link: https://dspace.bracu.ac.bd/xmlui/bitstream/handle/10361/24921/21371010_EEE.pdf

- ESP32 Based Implementation of Water Quality & Quantity Regulating System

Link: https://www.researchgate.net/publication/354882182_ESP32_Based_Implementation_of_Water_Quality_and_Quantity_Regulating_System

- IoT Based Water Quality Monitoring System using ESP32

- Link: https://www.ijraset.com/research-paper/iot-based-water-quality-monitoring-system-using-esp32

- Detection and Spatial Mapping of Mercury Contamination in Water Samples Using a Smart-Phone by M. D. Hossain, et al.

Link:  https://pmc.ncbi.nlm.nih.gov/articles/PMC3949663/

- Detection of Mercury in Water using Filter Paper Based Channel and Colorimetric-Android Readout by C. U. Yurddaş, et al.

Link: https://www.researchgate.net/publication/338833163_Detection_of_Mercury_in_Water_using_Filter_Paper_Based_Channel_and_Colorimetric-Android_Readout

- Rapid Removal of Mercury from Water by Novel MOF/PP Membranes by X. Li, et al. (Semantic Scholar/NIH, 2021): Rapid Removal of Mercury from Water by Novel MOF/PP Membranes by X. Li, et al.

Link: https://pdfs.semanticscholar.org/307b/ebe1b03d946f9e71704d10aa8b497f8fd98d.pdf

- Removal of mercury from water via phage virion-based biosorption

Link: https://www.researchgate.net/publication/385960276_An_environmental_fairytail_removal_of_mercury_from_water_via_phage_virion-based_biosorption

- Selective adsorption of mercury ion from water by a novel magnetic functionalized titanium-based MOF composite

Link: https://www.sciencedirect.com/science/article/abs/pii/S0021979723014911

- A critical review on the removal of mercury (Hg²⁺) from aqueous solution using nano adsorbents

Link: https://www.google.com/url?sa=i&source=web&rct=j&url=https://www.sciencedirect.com/science/article/abs/pii/S2215153223000405&ved=2ahUKEwjC_9_vz9yQAxW2BfsDHQ5yCDQQy_kOegQIBxAF&opi=89978449&cd&psig=AOvVaw1dYeaPOq-w--J3bcF-J5IX&ust=1762487541129000

- Fast and Efficient Removal of Mercury from Water Samples Using Magnetic Iron Oxide Nanoparticles Modified with 2-Mercaptobenzothiazole

Link: https://www.researchgate.net/publication/221746657_Fast_and_Efficient_Removal_of_Mercury_from_Water_Samples_Using_Magnetic_Iron_Oxide_Nanoparticles_Modified_with_2-Mercaptobenzothiazole

- Mercury Removal from Wastewater Using Cysteamine Functionalized Ultrafiltration Membranes

Link: https://www.google.com/url?sa=i&source=web&rct=j&url=https://pubs.acs.org/doi/10.1021/acsomega.0c02526&ved=2ahUKEwjC_9_vz9yQAxW2BfsDHQ5yCDQQy_kOegQIBxAK&opi=89978449&cd&psig=AOvVaw1dYeaPOq-w--J3bcF-J5IX&ust=1762487541129000

- Detection of Mercury in Water using Filter Paper Based Channel and Colorimetric-Android Readout

Link: https://www.researchgate.net/publication/338833163_Detection_of_Mercury_in_Water_using_Filter_Paper_Based_Channel_and_Colorimetric-Android_Readout